SUSPENSION GEOMETRY — HARDPOINTS 2027

Double A-arm (SLA), front and rear  |  FSAE 2027 -- PUCPR Racing

Sign conventions: negative camber = top of the wheel inboard; toe-in positive; positive force = tension. Lengths in mm, angles in degrees unless stated. Two coordinate frames appear in this document and every table names the one it uses — see Provenance below.

Generated 2026-10-12 by scripts/build_summary_doc.py from sla_geometry.py and steering_geometry.py. Do not hand-edit: change the config in those scripts and re-run. This document supersedes "Hardpoints Suspensão 2027", whose rate tables predated the upright-rotation sign fix (commit 0e7e524) and whose force tables assumed a different weight distribution from its own roll-stiffness section.

0. Provenance and frames

| Content | Produced by |
| --- | --- |
| Wishbone hardpoints, static KPIs, rates, roll, anti-geometry, leg forces | sla_geometry.py |
| Caster, outboard ball joints, tie rod, rack, bump steer, Ackermann, effort | steering_geometry.py |
| 3D kinematic solve behind the steering numbers | vdcore.geometry.solver.DWSolver |
| Composition, cross-checks, this document | scripts/geometry_summary.py + build_summary_doc.py |

| Frame | Axes | Used in |
| --- | --- | --- |
| DESIGN | y outboard, z up, x REARWARD | Sections 2 and 3 (front-view work) |
| ISO 8855 | X FORWARD, Y LEFT, Z up | Section 6 (model export) |

Conversion: X_iso = −x_rearward, Y_iso = ±y_outboard, Z_iso = z. In ISO 8855 the rear axle is at NEGATIVE X and right-hand corners are at NEGATIVE Y.

The rear toe link is a declared design_intent input (REAR_TOE_LINK_INBOARD / _OUTBOARD in geometry_summary.py), not a measured or synthesised value. The rear has no rack, so steering_geometry.py -- which covers the front axle only -- does not produce it. It is regenerated with every run, and its inboard Z was chosen to null the linear bump-steer rate; section 3c reports the solved result and how sharp that knob is.

1. Vehicle, stiffness and rules compliance

| Parameter | Value |
| --- | --- |
| Total / sprung mass | 315.0 / 270.0 kg |
| Wheelbase | 1540.0 mm |
| Front track / rear track | 1240.0 / 1200.0 mm |
| CG height / station | 320.0 mm / 847.0 mm aft of the front axle |
| Static front mass fraction | 45.0 % |
| Roll axis height at the CG | 46.0 mm |
| Roll moment arm | 274.0 mm |
| Target roll gradient | 1.00 °/g |
| Required roll stiffness | 725.7 N·m/° |
| Chassis torsional stiffness | 2177 (min) / 3629 (target) N·m/° |

The front mass fraction sets BOTH the roll axis height and the leg-force load cases in section 7. It is a single input; the two must never be allowed to disagree again.

| Status | Item | Value | Target |
| --- | --- | --- | --- |
| OK | Wheelbase | 1540 mm | 1525 to 10000 |
| OK | Narrow / wide track ratio | 96.8 % | 75 to 100 |
| OK | 60° tilt test | 1200 mm fitted | needs 1109 mm |

The tilt test uses the NARROWER track, which is the rear (1200 mm). Quoting that as "the track" hides the 1240 mm on the other axle.

2. Front suspension

Front-view points

DESIGN frame: y positive OUTBOARD, z positive up, origin at ground level on the car centreline. Two decimals are required — the integers printed in the superseded document could not reproduce its own FVIC or roll centre.

| Point | y [mm] | z [mm] |
| --- | --- | --- |
| Lower ball joint (LBJ) | 582.00 | 130.00 |
| Upper ball joint (UBJ) | 536.97 | 385.40 |
| LCA inboard | 175.00 | 117.38 |
| UCA inboard | 210.00 | 316.01 |
| FVIC (construction) | -880.00 | 84.68 |

Wishbones

| Parameter | Value |
| --- | --- |
| LCA / UCA length (front view) | 407.20 / 334.25 mm (ratio 0.821) |
| Outboard vertical separation | 255.40 mm |
| Inboard vertical separation | 198.63 mm |
| LCA inclination | 1.78° (falls from wheel to chassis) |
| UCA inclination | 11.98° (falls from wheel to chassis) |
| RC that would flatten the LCA | 53.73 mm |

Checks

| Status | Item | Value | Target |
| --- | --- | --- | --- |
| OK | Roll centre height | 35.00 mm | 20 to 70 |
| OK | FVSA length | 1500.00 mm | 1300 to 1700 |
| OK | Scrub radius | 15.08 mm | 5 to 25 |
| OK | KPI | 10.00 ° | 6 to 14 |
| OK | Kingpin length (front view) | 259.34 mm | 200 to 260 |
| OK | LCA length (front view) | 407.20 mm | 320 to 490 |
| OK | UCA / LCA ratio | 0.821 | 0.55 to 0.98 |
| OK | Camber gain | 0.0386 °/mm | 0.03 to 0.05 |
| OK | FVIC on the far side of the car | -880.00 mm | — |
| OK | Ball joints inside the rim | 130 / 385 mm | 80 to 410 |

Kingpin and arm lengths here are FRONT-VIEW projections — the right quantity for the swing-arm construction, but not the length that gets cut. Section 5 checks the true 3D members.

Rates about static

| Quantity | Value |
| --- | --- |
| Camber gain | -0.0386 °/mm |
| Roll centre migration | -0.1999 mm/mm |
| Half-track change | +0.0574 mm/mm |
| Camber at full bump | -2.52° |
| Camber at full droop | -0.58° |
| RC over the travel range | 30.7 to 40.7 mm |

Travel ±25 mm. Camber gain equals 57.2958 / FVSA = 0.0382 °/mm, the textbook value for a wheel turning about an instant centre 1500 mm away. Half-track change is POSITIVE: the contact patch moves outboard in bump.

At 1.5° of roll

| Quantity | Value |
| --- | --- |
| Outer wheel camber vs road | -0.65° (static -1.50°) |
| Inner wheel camber vs road | -2.39° |
| Roll centre height | 35.2 mm (design 35.0) |
| Roll centre lateral migration | -55.8 mm |
| Wheel travel at that roll | 16.23 mm |
| Outer camber in the useful window | OK (-2.5 to 0.0°) |

Longitudinal layout and anti-geometry

DESIGN frame, x positive REARWARD. Sweep and e/a as BUILT are in section 5, measured off the caster-corrected ball joints.

| Parameter | Value |
| --- | --- |
| LCA pickups x | -130.0 / 130.0 mm |
| UCA pickups x | -120.0 / 120.0 mm |
| LCA / UCA e/a ratio | see section 5 — measured as built (target ≤ 2) |
| Anti-dive | 7.50 % (target 0 to 30) |

3. Rear suspension

Front-view points

DESIGN frame: y positive OUTBOARD, z positive up, origin at ground level on the car centreline. Two decimals are required — the integers printed in the superseded document could not reproduce its own FVIC or roll centre.

| Point | y [mm] | z [mm] |
| --- | --- | --- |
| Lower ball joint (LBJ) | 558.60 | 130.00 |
| Upper ball joint (UBJ) | 519.69 | 390.34 |
| LCA inboard | 175.00 | 129.53 |
| UCA inboard | 210.00 | 328.85 |
| FVIC (construction) | -800.00 | 128.33 |

Wishbones

| Parameter | Value |
| --- | --- |
| LCA / UCA length (front view) | 383.60 / 315.74 mm (ratio 0.823) |
| Outboard vertical separation | 260.34 mm |
| Inboard vertical separation | 199.32 mm |
| LCA inclination | 0.07° (falls from wheel to chassis) |
| UCA inclination | 11.23° (falls from wheel to chassis) |
| RC that would flatten the LCA | 55.71 mm |

Checks

| Status | Item | Value | Target |
| --- | --- | --- | --- |
| OK | Roll centre height | 55.00 mm | 20 to 70 |
| OK | FVSA length | 1400.00 mm | 1300 to 1700 |
| OK | Scrub radius | 21.97 mm | 5 to 25 |
| OK | KPI | 8.50 ° | 3 to 10 |
| FAIL | Kingpin length (front view) | 263.23 mm | 200 to 260 |
| OK | LCA length (front view) | 383.60 mm | 320 to 490 |
| OK | UCA / LCA ratio | 0.823 | 0.55 to 0.98 |
| OK | Camber gain | 0.0411 °/mm | 0.03 to 0.05 |
| OK | FVIC on the far side of the car | -800.00 mm | — |
| OK | Ball joints inside the rim | 130 / 390 mm | 80 to 410 |

Kingpin and arm lengths here are FRONT-VIEW projections — the right quantity for the swing-arm construction, but not the length that gets cut. Section 5 checks the true 3D members.

Rates about static

| Quantity | Value |
| --- | --- |
| Camber gain | -0.0411 °/mm |
| Roll centre migration | -0.3380 mm/mm |
| Half-track change | +0.0922 mm/mm |
| Camber at full bump | -2.56° |
| Camber at full droop | -0.51° |
| RC over the travel range | 46.8 to 63.7 mm |

Travel ±25 mm. Camber gain equals 57.2958 / FVSA = 0.0409 °/mm, the textbook value for a wheel turning about an instant centre 1400 mm away. Half-track change is POSITIVE: the contact patch moves outboard in bump.

At 1.5° of roll

| Quantity | Value |
| --- | --- |
| Outer wheel camber vs road | -0.66° (static -1.50°) |
| Inner wheel camber vs road | -2.37° |
| Roll centre height | 54.6 mm (design 55.0) |
| Roll centre lateral migration | -56.3 mm |
| Wheel travel at that roll | 15.70 mm |
| Outer camber in the useful window | OK (-2.5 to 0.0°) |

Longitudinal layout and anti-geometry

DESIGN frame, x positive REARWARD. Sweep and e/a as BUILT are in section 5, measured off the caster-corrected ball joints.

| Parameter | Value |
| --- | --- |
| LCA pickups x | 1240.0 / 1440.0 mm |
| UCA pickups x | 1300.0 / 1460.0 mm |
| LCA / UCA e/a ratio | see section 5 — measured as built (target ≤ 2) |
| Anti-squat | 0.00 % (target 0 to 30) |

All pivot axes are exactly horizontal, so the side-view instant centre is at infinity and the anti-feature is identically zero — not approximately zero. That is a design decision to defend, not a check that happened to pass.

4. Steering (front axle)

Absent in its entirety from the superseded document. Caster and mechanical trail set steering feel and effort; bump steer is the tie rod's primary KPI.

| Parameter | Value |
| --- | --- |
| Caster angle | 5.00° |
| Mechanical trail | 21.43 mm |
| Scrub radius (3D) | 15.08 mm |
| Steering arm length | 80.00 mm |
| Tie rod length | 340.84 mm |
| Rack position (x rearward, z) | 30.0, 157.4 mm |
| Rack half length / max travel | 270.0 / ±38.0 mm |

| Quantity | Value |
| --- | --- |
| Bump steer (per side) | -0.00000 °/mm |
| Bump steer (total toe) | +0.00000 °/mm |
| Toe at full bump / droop (per side) | +0.158 / +0.160 ° |
| C-factor | -1.278 mm/° |
| Steering ratio | 4.58 : 1 |
| Max steer at stroke (outer / inner) | 27.06 / 39.65° |
| Ackermann at target steer | 70.0 % |
| Worst rod-end misalignment | 7.82° |

Bump steer is per side and total toe separately, because per-side versus total has caused confusion on the real car. The gradient is effectively zero at static: both tie rods aim at their own front-view instant centre. The residual is second order and the same sign at both ends of travel, so roll steer stays near zero while total toe rises slightly at full travel.

Steering effort (parking)

| Quantity | Value |
| --- | --- |
| Fz per front wheel | 695.3 N |
| Kingpin moment per wheel | 18.22 N·m |
| Rack force (both wheels) | 497.6 N |
| Steering wheel torque | 7.96 N·m |
| Rim force | 56.9 N |

Parking effort assumes a friction coefficient. PUCPR Racing has no TTC tyre data — this is a design_intent assumption, not a measured value.

5. Member lengths and sweep as built (true 3D)

Measured off the merged hardpoints in ISO 8855, after caster. These are the members that get cut, as opposed to the front-view projections checked in sections 2 and 3.

Front axle [FL]

| Status | Item | Value | Target |
| --- | --- | --- | --- |
| OK | LCA front leg | 424.49 mm | 320 to 490 |
| OK | LCA rear leg | 430.61 mm | 320 to 490 |
| OK | UCA front leg | 358.42 mm | 320 to 490 |
| OK | UCA rear leg | 352.34 mm | 320 to 490 |
| OK | Kingpin length, front view | 259.34 mm | 200 to 260 |
| FAIL | Kingpin length, TRUE 3D | 260.30 mm | 200 to 260 |

| Parameter | Value |
| --- | --- |
| LCA base / sweep as built | 260.00 / 10.06 mm (e/a 0.077) |
| UCA base / sweep as built | 240.00 / 12.28 mm (e/a 0.102) |
| Tie rod | 340.84 mm |

Rear axle [RL]

| Status | Item | Value | Target |
| --- | --- | --- | --- |
| OK | LCA front leg | 486.98 mm | 320 to 490 |
| OK | LCA rear leg | 396.42 mm | 320 to 490 |
| OK | UCA front leg | 396.60 mm | 320 to 490 |
| OK | UCA rear leg | 325.71 mm | 320 to 490 |
| FAIL | Kingpin length, front view | 263.23 mm | 200 to 260 |
| FAIL | Kingpin length, TRUE 3D | 263.23 mm | 200 to 260 |

| Parameter | Value |
| --- | --- |
| LCA base / sweep as built | 200.00 / 200.00 mm (e/a 2.000) |
| UCA base / sweep as built | 160.00 / 160.00 mm (e/a 2.000) |
| Tie rod | 379.07 mm |

Any leg flagged above is longer than the packaging window the front-view check passes it against. The swept rear legs matter most: e/a above 1 means the sweep roughly doubles the leg load, and buckling of a long compression member is outside this tool's scope. Someone has to check it.

6. Merged hardpoints (ISO 8855)

X POSITIVE FORWARD. Y POSITIVE LEFT. Z POSITIVE UP. Origin at the front axle centreline, ground plane, vehicle centreline. The rear axle is therefore at NEGATIVE X and right-hand corners at NEGATIVE Y. This is NOT the design frame used in sections 2 and 3.

FL

| Point | X [mm] | Y [mm] | Z [mm] |
| --- | --- | --- | --- |
| UCA_IN_FRONT | 120.00 | 210.00 | 321.66 |
| UCA_IN_REAR | -120.00 | 210.00 | 310.36 |
| UCA_OUT | -12.28 | 536.97 | 385.40 |
| LCA_IN_FRONT | 130.00 | 175.00 | 117.38 |
| LCA_IN_REAR | -130.00 | 175.00 | 117.38 |
| LCA_OUT | 10.06 | 582.00 | 130.00 |
| TIE_ROD_IN | -30.00 | 270.00 | 157.39 |
| TIE_ROD_OUT | 84.59 | 590.29 | 178.75 |
| WHEEL_CENTER * | 0.00 | 613.58 | 245.00 |
| CONTACT_PATCH * | 0.00 | 620.00 | 0.00 |

FR

| Point | X [mm] | Y [mm] | Z [mm] |
| --- | --- | --- | --- |
| UCA_IN_FRONT | 120.00 | -210.00 | 321.66 |
| UCA_IN_REAR | -120.00 | -210.00 | 310.36 |
| UCA_OUT | -12.28 | -536.97 | 385.40 |
| LCA_IN_FRONT | 130.00 | -175.00 | 117.38 |
| LCA_IN_REAR | -130.00 | -175.00 | 117.38 |
| LCA_OUT | 10.06 | -582.00 | 130.00 |
| TIE_ROD_IN | -30.00 | -270.00 | 157.39 |
| TIE_ROD_OUT | 84.59 | -590.29 | 178.75 |
| WHEEL_CENTER * | 0.00 | -613.58 | 245.00 |
| CONTACT_PATCH * | 0.00 | -620.00 | 0.00 |

RL

| Point | X [mm] | Y [mm] | Z [mm] |
| --- | --- | --- | --- |
| UCA_IN_FRONT | -1300.00 | 210.00 | 328.85 |
| UCA_IN_REAR | -1460.00 | 210.00 | 328.85 |
| UCA_OUT | -1540.00 | 519.69 | 390.34 |
| LCA_IN_FRONT | -1240.00 | 175.00 | 129.53 |
| LCA_IN_REAR | -1440.00 | 175.00 | 129.53 |
| LCA_OUT | -1540.00 | 558.60 | 130.00 |
| TIE_ROD_IN | -1440.00 | 175.00 | 169.00 |
| TIE_ROD_OUT | -1594.60 | 520.81 | 183.53 |
| WHEEL_CENTER * | -1540.00 | 593.58 | 245.00 |
| CONTACT_PATCH * | -1540.00 | 600.00 | 0.00 |

RR

| Point | X [mm] | Y [mm] | Z [mm] |
| --- | --- | --- | --- |
| UCA_IN_FRONT | -1300.00 | -210.00 | 328.85 |
| UCA_IN_REAR | -1460.00 | -210.00 | 328.85 |
| UCA_OUT | -1540.00 | -519.69 | 390.34 |
| LCA_IN_FRONT | -1240.00 | -175.00 | 129.53 |
| LCA_IN_REAR | -1440.00 | -175.00 | 129.53 |
| LCA_OUT | -1540.00 | -558.60 | 130.00 |
| TIE_ROD_IN | -1440.00 | -175.00 | 169.00 |
| TIE_ROD_OUT | -1594.60 | -520.81 | 183.53 |
| WHEEL_CENTER * | -1540.00 | -593.58 | 245.00 |
| CONTACT_PATCH * | -1540.00 | -600.00 | 0.00 |

* reference points, not hardpoints.

STATIC ALIGNMENT ENCODED HERE: camber -1.50°, toe +0.00° per side. CONTACT_PATCH sits directly below WHEEL_CENTER on all four corners, which is a zero-camber, zero-toe convention, while the rate and roll tables assume the design-intent static camber. Decide the convention once and apply it everywhere — see section 8.

7. Load case behind the leg forces

The superseded document printed leg forces with no stated load case, so they could not be checked or defended. These are the assumptions.

| Assumption | Value | Source |
| --- | --- | --- |
| Total mass | 315.0 kg | input |
| Front mass fraction | 45.0 % | input |
| CG height | 320.0 mm | estimate |
| Lateral friction coefficient | 1.50 | design_intent |
| Longitudinal friction coefficient | 1.40 | design_intent |
| Lateral acceleration | 1.50 g | design_intent |
| Lateral load transfer distribution (front share) | 0.45 | design_intent |

LLTD 0.45 against a front mass fraction of 0.45 makes lateral transfer proportional to axle load, so it introduces no balance shift of its own. That is a neutral BASELINE, not a tuned value — the balance target that would justify tuning away from it needs tyre data the team does not have.

| Axle | Static / wheel | Outer | Inner | Inner lifts at LLTD |
| --- | --- | --- | --- | --- |
| Front | 695.3 N | 1233.6 N | 157.0 N | 0.581 |
| Rear | 849.8 N | 1529.6 N | 170.0 N | 0.312 |

The usable band is LLTD 0.312 to 0.581. Outside it one inner wheel lifts at 1.50 g and every force in this section is void — the model loads a single outer wheel.

TWO LIMITS, to be repeated wherever the force table appears:

1. The friction coefficients are a TYRE assumption. PUCPR Racing has no TTC data, so every force inherits an unmeasured input.

2. There is no pushrod in this model. An upright on two A-arms and a tie rod has five constraints against six degrees of freedom; the sixth member is the pushrod. The solve closes the system by treating the UCA as a two-force member with no X component and enforcing only the moment about the roll axis, and it places the front ball joints at zero caster, so braking generates no kingpin moment and the tie rod carries nothing.

These are screening numbers. They are NOT a load set to size tubes from.

8. Open items

| Item | Effect | Consequence |
| --- | --- | --- |
| Front scrub if static camber were built in | 15.08 → 21.49 mm | still in window |
| Rear scrub if static camber were built in | 21.97 → 28.39 mm | leaves the 5 to 25 window |

The static camber / toe convention has to be decided once: either the export carries it and scrub radius moves, or it does not and the rate tables stop claiming it.

The front mass fraction is an unusual value for a rear-drive car and it drives both the roll-stiffness requirement and every leg force. Confirm it before the chassis is sized against it.

The rear toe link outboard point sits 54.6 mm behind the wishbone outboard ball joints, at Y = 521. It is an upright feature, not a chassis connection, so it stayed put when the inboard end moved forward. Bringing it ahead of the axle would be a rear-upright redesign and would flip the sign of toe change under longitudinal load.

Deliberately not computed here, and out of scope for this tool: wheel rates, motion ratio, ride frequency, damping; stress, deflection, fatigue, buckling; anything requiring tyre data until TTC data is acquired.

9. Charts

No figure in this revision. geometria.png (2026-08-26) predates the current geometry and nothing in the repository regenerates it, so it would show a superseded car next to current tables. The numbers it plotted — front-view construction, camber and roll centre vs travel, half-track change, roll behaviour — are all in sections 2 and 3 as solved values.